To	Copy

How to install driver for FBM 2180 in Windows 7

1	Revision history	1

2	document definition	1

3	installation procedure	2

# Revision history

| Issue | Date | Issued by | Modification |
| --- | --- | --- | --- |
| 1 | 2010-10-27 | TT-TH | Created |

# document definition

It has been found out that the procedure for installing the driver for FBM 2180 is a little bit different in Windows 7 compared to Windows XP and this document describes how to do in Win7.

If the RS232 connection on the FBM 2180 is used, no driver is needed since the FBM use a physical COM port on the PC.

NOTE: The driver only works in 32 bit Windows7 since 64 bit isn’t supported at all.

# installation procedure

- Install TM.

- When PC is restarted, connect the FBM 2180 to the PC.

- Windows will try to install the driver but will fail and there will be an error message about that the driver couldn’t be installed.

- Open the Device Manager and located the “Other devices -> FBM 2180”.

- Right click on “FBM 2180” and select “Update Driver Software”.

- The system will ask you from where the driver shall be installed. Select the second choice, “Browse my computer….”

- Browse to the TM CD and the “FBM2180” folder.

- Press “Next” to install the driver.

- The driver is installed.

- By looking in the device manager once again, we can see that only the driver USB to serial is installed and we need to do the same procedure once again to install the FBM 2180 driver itself.

- Right click on “USB Serial Port” and select “Update Driver Software”.

- Locate the “FBM2180” folder on the CD.

- Install the driver for FBM2180.

- It shall now look as below and the installation is finished.